Internet Area Working Group                                J. Chroboczek

Internet-Draft                               IRIF, Université Paris-Cité

Intended status: Standards Track                               W. Kumari

Expires: 17 September 2026                                   Google, LLC

T. Høiland-Jørgensen

Red Hat

16 March 2026

IPv4 routes with an IPv6 next hop

draft-ietf-intarea-v4-via-v6-07

Abstract

V4-via-v6 routing is a technique that uses IPv6 next-hop addresses

for routing IPv4 packets, and thus makes it possible to route IPv4

packets across a network where some routers have not been assigned

IPv4 addresses.  This document describes v4-via-v6 routing, and

defines related operational procedures, notably the origination of

ICMPv4 packets by nodes that might not have an IPv4 address.

About This Document

This note is to be removed before publishing as an RFC.

The latest revision of this draft can be found at

https://wkumari.github.io/draft-chroboczek-intarea-v4-via-v6/draft-

ietf-intarea-v4-via-v6.html.  Status information for this document

may be found at https://datatracker.ietf.org/doc/draft-ietf-intarea-

v4-via-v6/.

Discussion of this document takes place on the Internet Area Working

Group Working Group mailing list (mailto:int-area@ietf.org), which is

archived at https://mailarchive.ietf.org/arch/browse/int-area/.

Subscribe at https://www.ietf.org/mailman/listinfo/int-area/.

Source for this draft and an issue tracker can be found at

https://github.com/wkumari/draft-chroboczek-intarea-v4-via-v6.

Status of This Memo

This Internet-Draft is submitted in full conformance with the

provisions of BCP 78 and BCP 79.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF).  Note that other groups may also distribute

working documents as Internet-Drafts.  The list of current Internet-

Drafts is at https://datatracker.ietf.org/drafts/current/.

Chroboczek, et al.      Expires 17 September 2026               [Page 1]

Internet-Draft                  v4-via-v6                     March 2026

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

This Internet-Draft will expire on 17 September 2026.

Copyright Notice

Copyright (c) 2026 IETF Trust and the persons identified as the

document authors.  All rights reserved.

This document is subject to BCP 78 and the IETF Trust's Legal

Provisions Relating to IETF Documents (https://trustee.ietf.org/

license-info) in effect on the date of publication of this document.

Please review these documents carefully, as they describe your rights

and restrictions with respect to this document.  Code Components

extracted from this document must include Revised BSD License text as

described in Section 4.e of the Trust Legal Provisions and are

provided without warranty as described in the Revised BSD License.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   3

2.  Conventions and Definitions . . . . . . . . . . . . . . . . .   4

3.  Operation . . . . . . . . . . . . . . . . . . . . . . . . . .   4

3.1.  Structure of the routing table  . . . . . . . . . . . . .   5

3.2.  Operation of the forwarding plane . . . . . . . . . . . .   5

3.3.  Operation of routing protocols  . . . . . . . . . . . . .   5

4.  ICMP Considerations . . . . . . . . . . . . . . . . . . . . .   6

5.  Implementation Status . . . . . . . . . . . . . . . . . . . .   7

5.1.  Arista EOS  . . . . . . . . . . . . . . . . . . . . . . .   7

5.2.  The Babel routing protocol  . . . . . . . . . . . . . . .   7

5.3.  Linux . . . . . . . . . . . . . . . . . . . . . . . . . .   8

5.3.1.  Example:  . . . . . . . . . . . . . . . . . . . . . .   8

5.4.  Mikrotik RouterOS . . . . . . . . . . . . . . . . . . . .   8

5.4.1.  Example . . . . . . . . . . . . . . . . . . . . . . .   9

5.5.  Cisco NX-OS . . . . . . . . . . . . . . . . . . . . . . .   9

6.  Operational Considerations  . . . . . . . . . . . . . . . . .   9

7.  Security Considerations . . . . . . . . . . . . . . . . . . .  10

8.  IANA Considerations . . . . . . . . . . . . . . . . . . . . .  10

9.  References  . . . . . . . . . . . . . . . . . . . . . . . . .  10

9.1.  Normative References  . . . . . . . . . . . . . . . . . .  10

9.2.  Informative References  . . . . . . . . . . . . . . . . .  10

Acknowledgments . . . . . . . . . . . . . . . . . . . . . . . . .  12

Changes . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  12

Version 06-07 . . . . . . . . . . . . . . . . . . . . . . . . .  12

Version 05-06 . . . . . . . . . . . . . . . . . . . . . . . . .  12

Chroboczek, et al.      Expires 17 September 2026               [Page 2]

Internet-Draft                  v4-via-v6                     March 2026

Version 04-05 . . . . . . . . . . . . . . . . . . . . . . . . .  13

Version 03-04 . . . . . . . . . . . . . . . . . . . . . . . . .  13

Version 02-03 . . . . . . . . . . . . . . . . . . . . . . . . .  13

Version 01-02 . . . . . . . . . . . . . . . . . . . . . . . . .  13

Version 00-01 . . . . . . . . . . . . . . . . . . . . . . . . .  13

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . .  13

1.  Introduction

The dominant form of routing on the Internet is next-hop routing,

where a routing protocol constructs a routing table (or routing

information base, RIB) which is used by a forwarding process to

forward packets.  The routing table is a data structure that maps

network prefixes in each address family (IPv4 or IPv6) to next hops, pairs

of an outgoing interface and a neighbor's network address, for

example:

destination                      next hop

2001:db8:0:1::/64               eth0, fe80::1234:5678

203.0.113.0/24                  eth0, 192.0.2.1

When a packet is routed according to a given routing table entry, the

forwarding plane typically maps the next-hop address to a link-layer

address (a "MAC address") by using a neighbor discovery protocol (for

example the Neighbor Discovery protocol (ND) [RFC4861] in the case of

IPv6 over Ethernet, and the Address Resolution Protocol (ARP)

[RFC0826] in the case of IPv4 over Ethernet).  The link-layer address

is then used to construct the link-layer frames that encapsulate

forwarded packets.

It is apparent from the description above that there is no

fundamental reason why the destination prefix and the next-hop

address should be in the same address family: there is nothing

preventing an IPv6 packet from being routed through a next hop with

an IPv4 address (in which case the next hop's MAC address will be

obtained using ARP), or, conversely, an IPv4 packet from being routed

through a next hop with an IPv6 address.  (In fact, it is even

possible to store link-layer addresses directly in the next-hop entry

of the routing table, thus avoiding the use of an address resolution

protocol altogether, which was commonly done in networks using the

OSI protocol suite.)

This document focuses on the specific case of routing IPv4 packets

through an IPv6 next hop.  This case is particularly interesting,

since it makes it possible to build networks that have no IPv4

addresses except at the edges and still provide IPv4 connectivity to

edge hosts.  In addition, since an IPv6 next hop can use a link-local

address that is autonomously configured, the use of such routes

Chroboczek, et al.      Expires 17 September 2026               [Page 3]

Internet-Draft                  v4-via-v6                     March 2026

enables a mode of operation where the network core has no statically

assigned IP addresses of either family, which significantly reduces

the amount of manual configuration required.  (See also [RFC7404] for

a discussion of the issues involved with such an approach.)

A route towards an IPv4 prefix that uses an IPv6 next hop is called a

"v4-via-v6" route.  V4-via-v6 routing is not restricted to routers,

and could usefully be applied to hosts, but doing so would require

solving the issue of host configuration, for example by extending

either DHCPv4 or DHCPv6 to publish an IPv4 default route with an IPv6

next hop, which is out of scope for this document.

[RFC8950] discusses advertising of IPv4 Network Layer Reachability

Information (NLRI) with a next-hop address that belongs to the IPv6

protocol, but confines itself to how this is carried and advertised

in the BGP protocol.  This document, on the other hand, discusses the

concept of v4-via-v6 routes independently of any specific routing

protocol, their design and operational considerations, and the

implications of using them.

{ Editor note, to be removed before publication.  This document is

heavily based on draft-ietf-babel-v4viav6.  When draft-ietf-babel-

v4viav6 was going through IESG eval, Warren raised concerns that

something this fundamental deserved to be documented in a separate,

standalone document, so that it can be more fully discussed, and,

more importantly, referenced cleanly in the future.}

2.  Conventions and Definitions

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and

"OPTIONAL" in this document are to be interpreted as described in

BCP 14 [RFC2119] [RFC8174] when, and only when, they appear in all

capitals, as shown here.

3.  Operation

Next-hop routing is implemented by two separate components, the

routing protocol and the forwarding plane, that communicate through a

shared data structure, the routing table.

Chroboczek, et al.      Expires 17 September 2026               [Page 4]

Internet-Draft                  v4-via-v6                     March 2026

3.1.  Structure of the routing table

The routing table is a data structure that maps address prefixes to

next hops, pairs of the form (interface, address).  In traditional

next-hop routing, the routing table maps IPv4 prefixes to IPv4 next

hops, and IPv6 prefixes to IPv6 next hops.  With v4-via-v6 routing,

the routing table is extended so that an IPv4 prefix may map to

either an IPv6 or an IPv4 next hop.

Resolution may be recursive: the next hop may itself be a prefix that

requires further resolution to map to the outgoing interface and L2

address.  V4-via-v6 routing does not prevent recursive resolution.

3.2.  Operation of the forwarding plane

The forwarding plane is the part of the routing implementation that

is executed for every forwarded packet.  As a packet arrives, the

forwarding plane consults the routing table, selects a single route

matching the packet, and forwards the packet through the outgoing

interface to the associated next-hop address.

With v4-via-v6 routing, the address family of the next-hop address is

no longer determined by the address family of the prefix: since the

routing table may map an IPv4 prefix to either an IPv4 or an IPv6

next hop, the forwarding plane must be able to determine, on a per-

packet basis, which address resolution protocol (ARP for IPv4, ND for

IPv6) to consult.

3.3.  Operation of routing protocols

The routing protocol is the part of the routing implementation that

is executed asynchronously from the forwarding plane, and whose role

is to build the routing table.  Since v4-via-v6 routing is a

generalization of traditional next-hop routing, v4-via-v6 can

interoperate with existing routing protocols: a traditional routing

protocol produces a traditional next-hop routing table, which can be

used by an implementation supporting v4-via-v6 routing.

However, to use the additional flexibility provided by

v4-via-v6 routing, routing protocols need to be extended with the

ability to populate the routing table with v4-via-v6 routes when an

IPv4 address is not available or when the available IPv4 addresses

are not suitable for use as a next hop.

Some protocols already support the advertisement of IPv4 routes with

an IPv6 next hop, including Babel [RFC9229] and BGP [RFC8950].  Other

protocols advertise both IPv4 and IPv6 prefixes over a single

neighbor; these include:

Chroboczek, et al.      Expires 17 September 2026               [Page 5]

Internet-Draft                  v4-via-v6                     March 2026

*  Multi-Topology (MT) Routing in OSPF ([RFC4915])

*  Multi-Topology (MT) Routing in IS-IS ([RFC5120])

While both employ a common control plane, they use separate

data planes, and therefore don't implement v4-via-v6 routing.

4.  ICMP Considerations

The Internet Control Message Protocol (ICMPv4, or simply ICMP)

[RFC0792] is a protocol related to IPv4 that is primarily used to

carry diagnostic and debugging information.  ICMPv4 packets may be

originated by end hosts (e.g., the "destination unreachable, port

unreachable" ICMPv4 packet), but they may also be originated by

intermediate routers (e.g., most other kinds of "destination

unreachable" packets).

Some protocols deployed in the Internet rely on ICMPv4 packets sent

by intermediate routers.  Most notably, path MTU Discovery (PMTUd)

[RFC1191] is an algorithm executed by end hosts to discover the

maximum packet size that a route is able to carry.  While there exist

variants of PMTUd that are purely end-to-end [RFC4821], the variant

most commonly deployed on the Internet has a hard dependency on

ICMPv4 packets originated by intermediate routers: if intermediate

routers are unable to send ICMPv4 packets, PMTUd may lead to

persistent black-holing of IPv4 traffic.

A router must therefore be able to generate ICMP Destination

Unreachable messages ([RFC1812] Section 5.2.7.1).  The source address

of these messages must be one of the addresses assigned to the

outgoing interface; if no such address has been assigned, then one of

the other addresses assigned to the router, known as the "router-id",

must be used ([RFC1812] Section 4.3.2.4).

Routers implementing the mechanism described in this document do not

need to have any IPv4 addresses assigned to any of their interfaces,

and [RFC1812] does not specify what happens if no router-id has been

assigned.  If a router does not have any IPv4 addresses assigned, the

router MUST use the dummy address 192.0.0.8 as the source address of

outgoing ICMP packets ([RFC7600], Section 4.8, Requirement R-22).

Using the dummy address as the source of ICMPv4 packet causes a

number of drawbacks:

*  using the same address on multiple routers may hamper debugging

and fault isolation, e.g., when using the _traceroute_ utility

(but see [I-D.draft-ietf-intarea-extended-icmp-nodeid] for a

possible solution to this problem);

Chroboczek, et al.      Expires 17 September 2026               [Page 6]

Internet-Draft                  v4-via-v6                     March 2026

*  packets originating from 192.0.0.8 might be considered as spoofed

traffic and dropped by firewalls at network boundaries.

For these reasons, even if a router performs v4-via-v6 routing on all

interfaces, it MAY be assigned one or more IPv4 addresses.

5.  Implementation Status

(RFC Editor: please remove this section before publication.)

(This section records the status of known implementations of the

protocol defined by this specification at the time of writing, and is

based on a proposal described in RFC 7942.  The description of

implementations in this section are intended to assist the IETF in its

decision processes in progressing drafts to RFCs.  Please note that

the listing of any individual implementation here does not imply

endorsement by the IETF.  Furthermore, no effort has been spent to

verify the information presented here that was supplied by IETF

contributors.  This is not intended as, and must not be construed to

be, a catalog of available implementations or their features.

Readers are advised to note that other implementations may exist.)

5.1.  Arista EOS

Arista has supported static IPv4 routes with IPv6 next hops since

EOS-4.30.1.

5.2.  The Babel routing protocol

As noted above, this document is heavily based on RFC9229 (nee draft-

ietf-babel-v4viav6), and this functionality is supported by babeld.

Pasted below is email sent to the babel mailing list (archived at

https://mailarchive.ietf.org/arch/msg/babel/

QtFi3F4TFfF7fXXlkHSpEnuT44Y/)

An IPv4 route across three nodes with IPv6 addresses only:

$ ip route show 10.0.0.2

10.0.0.2 via inet6 fe80::216:3eff:fe00:1 dev lxcbr0 proto babel onlink

Here's how it's logged by babeld:

10.0.0.2/32 from 0.0.0.0/0 metric 384 (384) refmetric 288 id

02:16:3e:ff:fe:9a:5e:22 seqno 36425 chan (255) age 15 via lxcbr0 neigh

fe80::216:3eff:fe00:1 (installed)

Traceroute is a little confusing:

Chroboczek, et al.      Expires 17 September 2026               [Page 7]

Internet-Draft                  v4-via-v6                     March 2026

$ traceroute 10.0.0.2

traceroute to 10.0.0.2 (10.0.0.2), 30 hops max, 60 byte packets

1  192.0.0.8 (192.0.0.8)  0.079 ms  0.019 ms  0.014 ms

2  192.0.0.8 (192.0.0.8)  0.040 ms  0.023 ms  0.042 ms

3  192.0.0.8 (192.0.0.8)  0.061 ms  0.030 ms  0.030 ms

4  10.0.0.2 (10.0.0.2)  0.060 ms  0.040 ms  0.039 ms

PMTUD works fine (thanks to Toke):

19:58:47.402871 IP 192.168.0.27.60046 > 10.0.0.2.22: Flags [.],\

seq 33:1481, ack 33, win 502, options [nop,nop,TS val 917354570\

ecr 1849974691], length 1448

19:58:47.402874 IP 192.168.0.27.60046 > 10.0.0.2.22: Flags [P.],\

seq 1481:1537, ack 33, win 502, options [nop,nop,TS val 917354570\

ecr 1849974691], length 56

19:58:47.402906 IP 192.0.0.8 > 192.168.0.27: ICMP 10.0.0.2 \

unreachable- need to frag (mtu 1420), length 556

19:58:47.402919 IP 10.0.0.2.22 > 192.168.0.27.60046: Flags [.],\

ack 33, win 509, options [nop,nop,TS val 1849974692 \

ecr 917354569,nop,nop,sac 1 {1481:1537}], length 0

19:58:47.402934 IP 192.168.0.27.60046 > 10.0.0.2.22: Flags [.], \

seq 33:1401, ack 33, win 502, options [nop,nop,TS val 917354570 \

ecr 1849974692], length 1368

-- Juliusz

5.3.  Linux

Linux has supported v4-via-v6 routes since kernel version 5.2,

released on 2019-07-07.

5.3.1.  Example:

rincewind ~ #

ip -4 r a 192.0.2.23/32 via inet6 2001:db8::2342

rincewind ~ # ip r s 192.0.2.23/32

192.0.2.23 via inet6 2001:db8::2342 dev wlp36s0.25

5.4.  Mikrotik RouterOS

Mikrotik RouterOS has supported v4-via-v6 routes since (at least)

version 7.11beta2

{Editor note: I'm not sure when support was added.  I tested this in

Version 7.11beta2, and it worked there, but I believe that this

functionality has existed for a while.  I'll try to find out when it

was added.}

Chroboczek, et al.      Expires 17 September 2026               [Page 8]

Internet-Draft                  v4-via-v6                     March 2026

5.4.1.  Example

[wkumari@Dulles-CCR] /ip/route> print

Flags: D - DYNAMIC; I - INACTIVE, A - ACTIVE; c - CONNECT, s - STATIC,

d -DHCP, v - VPN; H - HW-OFFLOADED

Columns: DST-ADDRESS, GATEWAY, DISTANCE

#      DST-ADDRESS       GATEWAY                             DISTANCE

0  As  192.0.2.0/24      fe80::201:5cff:feb2:1646%1_Comcast         1

5.5.  Cisco NX-OS

Cisco NX-OS has supported v4-via-v6 routes "for more than 8 years" --

Krishnaswamy Ananthamurthy

6.  Operational Considerations

V4-via-v6 routing makes it easy to route IPv4 traffic across

interfaces where IPv4 addresses are not be configured, and therefore

reduces the number of IPv4 addresses used and

thus simplify the management of double-stack networks.  Since it

promises IPv4 routing essentially "for free" once IPv6 addressing has

been set up, it has the potential to accelerate the

deployment of IPv6.

Just like any other extension to an already deployed technology, it

requires also changes how it is being operated as well.  Even though v4-via-v6

routes are similar in structure to traditional next-hop routes,

some monitoring protocols and management tools

requires adjustment. To take IPFIX [RFC7011] as example for monitoring forward and data plane. Each IPFIX data and template record for IPv4 flows should contain IE62 ipNextHopIPv6Address to describe IPv6 next hop. Deployment of v4-via-v6 routing in a network

requires testing and potentially updating of all tools and scripts

that manipulate or examine routes.

V4-via-v6 routing encourages a model of deployment where some routers

have no IPv4 addresses even though they forward IPv4 traffic.  Such

routers make debugging of IPv4 routing issues more

difficult, because the output of the _traceroute_

utility has less informative than otherwise (see

Section Section 4).  Even if the procedures described in

[I-D.draft-ietf-intarea-extended-icmp-nodeid] are deployed on all

such routers, older versions of _traceroute_ will not be able to

interpret the additional information.  Network administrators might

want to provision IPv4 addresses on all routers to simplify

debugging.

Chroboczek, et al.      Expires 17 September 2026               [Page 9]

Internet-Draft                  v4-via-v6                     March 2026

7.  Security Considerations

The techniques described in this document make routing more flexible

by allowing IPv4 routes to propagate across a section of a network

that has only been assigned IPv6 addresses.  This additional

flexibility might invalidate otherwise reasonable assumptions made by

network administrators, which could potentially cause security

issues.

For example, if an island of IPv4-only hosts is separated from the

IPv4 Internet by routers that have not been assigned IPv4 addresses,

a network administrator might reasonably assume that the IPv4-only

hosts are unreachable from the IPv4 Internet.  This assumption is

broken if the intermediary routers implement v4-via-v6 routing, which

might make the IPv4-only hosts reachable from the IPv4 Internet.  If

this is not desirable, then the network administrator must filter out

the undesirable traffic in the forwarding plane by implementing

suitable packet filtering rules.

8.  IANA Considerations

No IANA actions are requested by this document.

9.  References

9.1.  Normative References

[RFC1812]  Baker, F., Ed., "Requirements for IP Version 4 Routers",

RFC 1812, DOI 10.17487/RFC1812, June 1995,

<https://www.rfc-editor.org/rfc/rfc1812>.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119,

DOI 10.17487/RFC2119, March 1997,

<https://www.rfc-editor.org/rfc/rfc2119>.

[RFC8174]  Leiba, B., "Ambiguity of Uppercase vs Lowercase in RFC

2119 Key Words", BCP 14, RFC 8174, DOI 10.17487/RFC8174,

May 2017, <https://www.rfc-editor.org/rfc/rfc8174>.

9.2.  Informative References

Chroboczek, et al.      Expires 17 September 2026              [Page 10]

Internet-Draft                  v4-via-v6                     March 2026

[I-D.draft-ietf-intarea-extended-icmp-nodeid]

Fenner, B. and R. Thomas, "Adding Extensions to ICMP

Errors for Originating Node Identification", Work in

Progress, Internet-Draft, draft-ietf-intarea-extended-

icmp-nodeid-04, 19 August 2025,

<https://datatracker.ietf.org/doc/html/draft-ietf-intarea-

extended-icmp-nodeid-04>.

[RFC0792]  Postel, J., "Internet Control Message Protocol", STD 5,

RFC 792, DOI 10.17487/RFC0792, September 1981,

<https://www.rfc-editor.org/rfc/rfc792>.

[RFC0826]  Plummer, D., "An Ethernet Address Resolution Protocol: Or

Converting Network Protocol Addresses to 48.bit Ethernet

Address for Transmission on Ethernet Hardware", STD 37,

RFC 826, DOI 10.17487/RFC0826, November 1982,

<https://www.rfc-editor.org/rfc/rfc826>.

[RFC1191]  Mogul, J. and S. Deering, "Path MTU discovery", RFC 1191,

DOI 10.17487/RFC1191, November 1990,

<https://www.rfc-editor.org/rfc/rfc1191>.

[RFC4821]  Mathis, M. and J. Heffner, "Packetization Layer Path MTU

Discovery", RFC 4821, DOI 10.17487/RFC4821, March 2007,

<https://www.rfc-editor.org/rfc/rfc4821>.

[RFC4861]  Narten, T., Nordmark, E., Simpson, W., and H. Soliman,

"Neighbor Discovery for IP version 6 (IPv6)", RFC 4861,

DOI 10.17487/RFC4861, September 2007,

<https://www.rfc-editor.org/rfc/rfc4861>.

[RFC4915]  Psenak, P., Mirtorabi, S., Roy, A., Nguyen, L., and P.

Pillay-Esnault, "Multi-Topology (MT) Routing in OSPF",

RFC 4915, DOI 10.17487/RFC4915, June 2007,

<https://www.rfc-editor.org/rfc/rfc4915>.

[RFC5120]  Przygienda, T., Shen, N., and N. Sheth, "M-ISIS: Multi

Topology (MT) Routing in Intermediate System to

Intermediate Systems (IS-ISs)", RFC 5120,

DOI 10.17487/RFC5120, February 2008,

<https://www.rfc-editor.org/rfc/rfc5120>.

[RFC7404]  Behringer, M. and E. Vyncke, "Using Only Link-Local

Addressing inside an IPv6 Network", RFC 7404,

DOI 10.17487/RFC7404, November 2014,

<https://www.rfc-editor.org/rfc/rfc7404>.

Chroboczek, et al.      Expires 17 September 2026              [Page 11]

Internet-Draft                  v4-via-v6                     March 2026

[RFC7600]  Despres, R., Jiang, S., Ed., Penno, R., Lee, Y., Chen, G.,

and M. Chen, "IPv4 Residual Deployment via IPv6 - A

Stateless Solution (4rd)", RFC 7600, DOI 10.17487/RFC7600,

July 2015, <https://www.rfc-editor.org/rfc/rfc7600>.

[RFC8950]  Litkowski, S., Agrawal, S., Ananthamurthy, K., and K.

Patel, "Advertising IPv4 Network Layer Reachability

Information (NLRI) with an IPv6 Next Hop", RFC 8950,

DOI 10.17487/RFC8950, November 2020,

<https://www.rfc-editor.org/rfc/rfc8950>.

[RFC9229]  Chroboczek, J., "IPv4 Routes with an IPv6 Next Hop in the

Babel Routing Protocol", RFC 9229, DOI 10.17487/RFC9229,

May 2022, <https://www.rfc-editor.org/rfc/rfc9229>.

Acknowledgments

This document is based on [RFC9229], which was produced by the IETF

Babel working group.

We are grateful to Joe Abley, Krishnaswamy Ananthamurthy, Vint Cerf,

Joe Clarke, Lorenzo Colitti, Bill Fenner, Tobias Fiebig, John

Gilmore, Bob Hinden, Jen Linkova, David Lamparter, Gyan Mishra, tom

petch, Herbie Robinson, Behcet Sarikaya, David Schinazi, Ole Troan,

and Éric Vyncke for helpful comments and suggestions about this

document.

Changes

This section is to be removed before publication, and the primary

change log is the git repository.  This is just a place to note some

of the more substantive changes.

Version 06-07

*  Made the reference to RFC 7600 informative.

*  Downgraded the suggestion to assign an IPv4 address from SHOULD to

MAY.

*  Changed the first author's affiliation (but don't get used to it,

they'll likely rename us again).

Version 05-06

*  Rewrote abstract.

*  Added RFC 7942 boilerplate.

Chroboczek, et al.      Expires 17 September 2026              [Page 12]

Internet-Draft                  v4-via-v6                     March 2026

*  Added positive factors to Operational Considerations.

*  Editorial changes.

Version 04-05

*  Minor editorial changes.

Version 03-04

*  Added a section about operational considerations.

*  Made it clear that ARP/ND are not necessarily used.

*  Removed any mention of v4-only, since it's not quite correct that

v4-via-v6 is v4-only.

Version 02-03

*  Warren is a smart guy, but he still pushed a branch instead of the

main one, so -03 is actually what -02 should have been.

Version 01-02

*  Addressed comments from Vint and Jen.

Version 00-01

*  Added note that this works just as well for IPv6 routes with an

IPv4 next hop. (Éric Vyncke)

*  Cisco NX-OS has supported v4-via-v6 routes "for more than 8 years"

(Krishnaswamy Ananthamurthy)

*  Mention recursive next hops, and that the next hop may be a

prefix.  (Krishnaswamy Ananthamurthy)

*  Hosts are routers too!  (David Lamparter)

*  Removed the claim that it's mainly a UI issue.

Authors' Addresses

Juliusz Chroboczek

IRIF, Université Paris-Cité

Case 7014

75205 Paris Cedex 13

France

Chroboczek, et al.      Expires 17 September 2026              [Page 13]

Internet-Draft                  v4-via-v6                     March 2026

Email: jch@irif.fr

Warren Kumari

Google, LLC

Email: warren@kumari.net

Toke Høiland-Jørgensen

Red Hat

Email: toke@toke.dk

Chroboczek, et al.      Expires 17 September 2026              [Page 14]